# PRISM LiDAR Cloud — Positional Accuracy Report

PRISM LiDAR Cloud — Survey-grade automated LiDAR analysis

DRAFT — FOR PSM REVIEW (not a sealed survey document)

| Report metadata |  |
| --- | --- |
| Site / project | Downtown Washington, DC (sample) |
| Input dataset | OpenDataDC 2020 airborne LiDAR (public domain) |
| Run timestamp (UTC) | 2026-07-03T20:19:13+00:00 |
| Run id | - |
| Operator | PRISM LiDAR Cloud |
| Engine version | PRISM·lidarcloud v5_4_10 |
| Processing package version | 6.0.0-dev |
| Engine file | - |
| Report builder (docx) | sap.report.docx 1.5.0 |

# 1. Project & Location

| Project / location |  |
| --- | --- |
| Horizontal CRS | EPSG:26985 (NAD83 / Maryland State Plane (EPSG:26985)) |
| Vertical datum | NAVD88 (GEOID18) |
| Vertical EPSG | 5703 |
| Vertical epoch | - |
| Orthometric / geoid | orthometric (GEOID18) |
| Linear units | metre |
| Area processed (km² / ac) | 0.3563 km² (88.0 ac) |
| Area processed (m² / ft²) | 356337.00 m² (88.053 ac) |
| Extent | Not available |

# 2. Acquisition & Sensor

| Acquisition / sensor |  |
| --- | --- |
| Acquisition date | Not recorded |
| System identifier | Not recorded |
| Generating software | Not recorded |
| Point data format | Not recorded |
| LAS version | Not recorded |
| Scan angle range (deg) | Not recorded |
| GPS time range | Not recorded |

Return-number counts: Not recorded.

# 3. Processing Settings

| Processing setting | Value |
| --- | --- |
| Engine version | PRISM·lidarcloud v5_4_10 |
| Engine file | - |
| DEM resolution | 0.50 m (19.7 in) |
| DSM resolution | - |

Pipeline summary

The bare-earth DEM, surface model (DSM) and canopy height model (CHM) are produced by the PRISM·lidarcloud processing engine (proprietary). The DEM is the classified bare-ground surface at the published cell size; the DSM is the top surface (vegetation, structures and bare ground); the CHM is the height of features above the ground (CHM = DSM − DEM). A classified point cloud and a per-cell DEM uncertainty layer are delivered alongside the rasters.

Datum & geoid handling

Products are reported in NAVD88 (GEOID18) (orthometric), derived from the WGS84 ellipsoidal input by applying the GEOID18 geoid model (H = h - N) per cell.

# 4. Products & Descriptions

| Product | Description |
| --- | --- |
| DEM — bare-earth (GeoTIFF + LAS) | Digital Elevation Model of the classified bare-ground surface, at the published cell size; the survey deliverable for terrain, hydrology and volume work. |
| DSM — surface model (GeoTIFF + LAS) | Digital Surface Model — the top surface (vegetation, buildings and bare ground) at the published cell size. |
| CHM — canopy height model (GeoTIFF + LAS) | Canopy Height Model = DSM - DEM; height of vegetation/structures above the bare earth, for forestry, biomass and obstruction analysis. |
| DEM uncertainty (GeoTIFF) | Per-cell uncertainty index (0 = data-supported ground, 1 = unsupported / interpolated); drives the percent-within-tolerance reliability map. |
| Classified point cloud (LAS/LAZ) | Full point cloud with ASPRS semantic classes (ground, low/med/high vegetation, building, water, wire), in ellipsoidal-Z and height-above-DEM variants. |
| AOI extent (KML / SHP / GeoJSON) | Processing-area boundary polygon for the delivered products, for GIS overlay and coverage verification. |
| CAD deliverables (DXF / LandXML) | Survey-CAD surfaces — DXF 3D contours/points and LandXML TIN, in the project's State Plane zone (ftUS) with NAVD88 (GEOID18) elevations — for ingest into civil/CAD workflows. |

# 5. Product Metrics & Summary Statistics

Dataset

| Metric | Value |
| --- | --- |
| Points (input) | 3,237,983 |
| Point density (pts/m² / pts/ft²) | 9.09 pts/m² (0.84 pts/ft²) |
| Ground-point density (pts/m² / pts/ft²) | 2.06 pts/m² (0.19 pts/ft²) |
| Interpolated DEM cells (no ground return) | 70.91 % |
| Ground fraction | - |
| Area (km² / ac) | 0.3563 km² (88.0 ac) |

DEM uncertainty summary

| Metric | Value |
| --- | --- |
| Tolerance (m / ft) | 0.150 m (0.492 ft) |
| % within tolerance | 87.28 % |
| Valid cells | 1,425,348 |
| Mean index (m / ft) | 0.115 m (0.377 ft) |
| Median index (m / ft) | 0.000 m (0.000 ft) |
| p95 index (m / ft) | 1.000 m (3.281 ft) |
| Max index (m / ft) | 1.000 m (3.281 ft) |
| Source | DEM_uncertainty.tif |

Accuracy headline

Not available — no surveyed checkpoints; see the 3DEP vertical-agreement block in the Accuracy & Validation section.

# 6. Accuracy & Validation

No surveyed checkpoints were supplied; a formal ASPRS Edition 1 / Edition 2 accuracy statement could not be computed. Where available, surface-to-surface agreement against the USGS 3DEP reference surface is reported below.

## Vertical agreement vs USGS 3DEP reference surface

| 3DEP agreement | Value |
| --- | --- |
| Samples (n) | 40 |
| Signed mean bias (m / ft) | 0.115 m (0.377 ft) (DEM - 3DEP) |
| RMSEz (m / ft) | 0.400 m (1.312 ft) |
| Std dev (m / ft) | 0.383 m (1.257 ft) |
| Min / max residual (m / ft) | -1.082 m (-3.550 ft) / 1.266 m (4.154 ft) |
| NVA-style 95% (m / ft) | 0.785 m (2.575 ft) (1.96 x RMSEz) |

EPQS = NAVD88 orthometric vs DEM = WGS84 ellipsoidal: the MEAN ≈ geoid offset (~−27 m in FL), and std / rmse_debiased is the true agreement (also limited by 3DEP's ~1 m resolution).  Confidence-gated (DEM uncertainty index <= 0.00) + spatially stratified across the AOI.  USGS 3DEP is an independent reference SURFACE, not surveyed checkpoints — these are surface-to-surface agreement statistics, NOT a formal ASPRS NVA/VVA.

# Point classification summary

ASPRS semantic classes assigned to the delivered point cloud (final_ellipsoidal_z_merged.las). 3,267,073 classified points.

| Class | Points | % |
| --- | --- | --- |
| High veg (5) | 1,700,658 | 52.05 % |
| Ground (2) | 734,500 | 22.48 % |
| Med veg (4) | 364,193 | 11.15 % |
| Low veg (3) | 241,323 | 7.39 % |
| Building (6) | 121,742 | 3.73 % |
| Water (9) | 68,906 | 2.11 % |
| Unclassified (1) | 35,748 | 1.09 % |
| Noise (7) | 3 | 0.00 % |

# Horizontal Alignment — Co-registration

Reference surface: USGS 3DEP (NAVD88, seamless best-available). The planimetric shift (and any residual vertical bias) of the DEM relative to the reference was recovered by Nuth & Kääb (2011) co-registration over sloped, multi-aspect terrain.

| Co-registration | Value |
| --- | --- |
| Resolvable | Yes |
| Shift east (dx) | -0.135 m (-0.443 ft) |
| Shift north (dy) | 0.916 m (3.005 ft) |
| Shift magnitude | 0.926 m (3.038 ft) @ 351.6° |
| Residual vertical bias (dz) | 0.026 m (0.086 ft) |
| Sigma XY / cross-track | 0.001 m (0.004 ft) / 0.001 m (0.004 ft) |
| dh NMAD (before → after) | 0.140 m (0.460 ft) → 0.118 m (0.387 ft) |
| dh RMSE (before → after) | 0.371 m (1.218 ft) → 0.369 m (1.212 ft) |
| Iterations | 4 (converged) |
| Inlier cells | 857,219 |
| Sloped fraction | 0.681 |
| Aspect octants used | 8 / 8 |

Estimate only — the shift was NOT applied; products remain in their original position. The figures above quantify the residual misalignment.

# 7. Figures

Bare-earth DEM — shaded relief (illumination 315°, 45°)

Digital Surface Model (DSM) — shaded relief

Canopy Height Model (CHM) — shaded relief

DEM uncertainty index — percent-within-tolerance map

# 10. Methodology

Pipeline

The bare-earth DEM, surface model (DSM) and canopy height model (CHM) are produced by the PRISM·lidarcloud processing engine (proprietary). The DEM is the classified bare-ground surface at the published cell size; the DSM is the top surface (vegetation, structures and bare ground); the CHM is the height of features above the ground (CHM = DSM − DEM). A classified point cloud and a per-cell DEM uncertainty layer are delivered alongside the rasters.

Datum & geoid handling

Products are reported in NAVD88 (GEOID18) (orthometric), derived from the WGS84 ellipsoidal input by applying the GEOID18 geoid model (H = h - N) per cell.

Vertical accuracy method

No surveyed vertical checkpoints were supplied, so a formal ASPRS NVA/VVA was not computed. Vertical agreement is instead reported against the USGS 3DEP reference surface (DEM minus 3DEP): signed mean bias, RMSEz, std, min/max and an NVA-style 95% (1.96 x RMSEz). This is surface-to-surface agreement against an independent reference surface, NOT a formal ASPRS checkpoint accuracy.

Checkpoint method

No control file was supplied; checkpoint analysis was not performed.

3DEP / EPQS method

An independent cross-check was performed against the USGS 3DEP Elevation Point Query Service (EPQS). Both the DEM and EPQS are NAVD88 orthometric, so the mean difference is the true vertical bias (no datum offset); the debiased RMSE / standard deviation is the agreement, itself bounded by 3DEP's ~1 m native resolution.

Limitations

Accuracy figures are bounded by the number and distribution of checkpoints and by the reference's own accuracy (per Edition-2 survey-error propagation). Datum shifts reported here, when not derived from a rigorous per-cell geoid grid, are first-order (constant bias or planar tilt) approximations. Vegetated vertical accuracy does not assume a normal error distribution. 3DEP cross-checks are limited by the reference DEM resolution.

Responsible charge

This positional-accuracy assessment was prepared under the responsible charge of a licensed Professional Surveyor and Mapper (PSM). The figures above are derived solely from the supplied control/checkpoint data and the delivered geospatial products; the signing surveyor affirms the methodology and results upon final review and signature.

# 11. Provenance

| Provenance | Value |
| --- | --- |
| Engine | PRISM·lidarcloud v5_4_10 |
| Processing package (integration) | 6.0.0-dev |
| run | 2026-07-03T20:19:13+00:00 |
| Report builder (docx) | sap.report.docx 1.5.0 |

Module versions

| Module | Version |
| --- | --- |
| bench.classification | 1.0.0 |
| bench.dep3 | 1.0.0 |
| bench.dep3_aggregate | 1.0.0 |
| bench.dep3_sample | 1.0.0 |
| bench.dep3_score | 1.0.0 |
| bench.isprs | 1.0.0 |
| breakline.detect | 1.0.0 |
| breakline.vectorize | 1.0.0 |
| cad | 1.0.0 |
| cad.crs | 1.0.0 |
| cad.dxf | 1.0.0 |
| cad.landxml | 1.2.0 |
| cad.pipeline | 1.2.0 |
| cad.stamp | 1.2.0 |
| cad.surface | 1.0.0 |
| cad.vertical | 1.0.0 |
| change.core | 1.1.0 |
| color | 1.2.1 |
| color.awss3 | 1.0.0 |
| color.cog | 1.0.0-dev |
| color.consistency | 1.1.0 |
| color.drape | 1.0.0 |
| color.fusion | 1.0.0-dev |
| color.laswrite | 1.0.0 |
| color.mosaic | 1.0.0 |
| color.raster | 1.6.1 |
| color.reflectance | 1.0.0 |
| color.registration | 1.0.0 |
| color.rgbtiff | 1.0.0-dev |
| color.selection | 1.0.0 |
| color.sources | 1.2.0 |
| color.stac | 1.0.0-dev |
| export | 1.2.0 |
| hydro.watermask | 1.0.0 |
| ign.fetch_lidarhd_urban | 1.0.0 |
| report.density | 1.1.0 |
| report.docx | 1.5.0 |
| report.facts | 1.4.0 |
| report.fgdc | 1.3.0 |
| report.narrate | 1.1.1 |
| report.pdf | 1.11.0 |
| report.units | 1.0.0 |
| stats.robust | 1.0.0 |
| survey.geoid | 1.1.0 |
| usgs.ept | 1.0.0 |
| validate.adjust | 1.2.0 |
| validate.asprs | 1.1.0 |
| validate.checkpoints | 1.3.1 |
| validate.coregister | 1.0.0 |
| validate.demdiff | 1.0.0 |
| validate.dep3raster | 1.0.0 |
| validate.epqs | 1.1.1 |
| vector.extent | 1.0.0 |
| vector.io | 1.0.0 |

All figures in this report are computed deterministically by the PRISM·lidarcloud processing engine from the inputs above (no clock, no randomness); the report renderer writes those values verbatim and computes none of the reported accuracy statistics itself. This document is a DRAFT until reviewed, signed and sealed by the Professional Surveyor & Mapper (PSM) / Professional Land Surveyor (PLS) of record under the licensing statutes and rules of the jurisdiction in which the survey is performed.